# 10 Popular Modules/Blocks Optimized with Bit Operations

Examples and concise code snippets

These are common hardware and low-level software patterns where bitwise operations provide a cleaner, faster, or more elegant implementation than arithmetic or iterative logic.

# 1. One-hot detector

Check whether exactly one bit is set. Used in arbiters, FSM encoding checks, and interrupt masks.

x != 0 && ((x & (x - 1)) == 0)

# 2. Power-of-two detector

Same core identity as one-hot. Useful for alignment logic, cache geometry checks, and buffer sizing.

x != 0 && ((x & (x - 1)) == 0)

# 3. Round-up division by power of two

Replace general division when the divisor is 2^k. Useful for packet chunking, tiling, and allocation sizing.

// ceil(x / 8)

(x + 7) >> 3

// general form for divisor 2^k

(x + (1 << k) - 1) >> k

# 4. Modulo by power of two

Replace x % 2^k with a mask. Common in circular buffers, address wrapping, and cache indexing.

// x % 8

x & 7

// general form

x & ((1 << k) - 1)

# 5. Multiply by powers of two

Use shifts for scaling or address generation.

// x * 8

x << 3

# 6. Divide by powers of two

Use shifts for unsigned scaling and indexing. Signed values need care because of rounding behavior.

// x / 8

x >> 3

# 7. Isolate lowest set bit

Very common primitive for priority selection and bitmap logic.

x & -x

# 8. Clear lowest set bit

Useful for iterating through active bits or removing one request at a time.

x & (x - 1)

# 9. Bit reversal / byte swap / field packing

Use masks and shifts or direct bit indexing instead of loops with complex conditionals. Common in FFT addressing, protocol conversion, CRC paths, and endianness conversion.

// bit reversal idea

y[7 - i] = x[i]

# 10. Gray code conversion

Standard low-transition encoding used in FIFO pointers and counters.

g = b ^ (b >> 1)

# Interview-popular examples

Especially common in interviews: one-hot / power-of-two check, round-up division by power of two, modulo by power of two, isolate lowest set bit, Gray code conversion, and alignment with masks.

Strong interview habit: state the assumption behind the bit trick, such as “the divisor is a power of two” or “this is for unsigned arithmetic.”